Android Application to test hearing impairments in adults

By

Junaid Malik

This report is submitted in partial fulfilment of the requirements of the B.Sc. Honours Degree in Networking Applications & Services, of the Dublin Institute of Technology

June 2nd, 2015

Supervisor: Dr Ray Lynch

School of Electrical and Electronic Engineering

Year Coordinator:

Joseph Kellegher				DT080B/4			Report

Declaration

I, the undersigned, declare that this report is entirely my own written work, except where otherwise accredited, and that it has not been submitted for a degree or award to any other university of institution.

Signed:	________________________

Date:		________________________

Contents

Acknowledgments	6

Abstract	7

Chapter 1 Introduction	8

1.1 How we hear sound?	10

1.2 The Audiogram	11

1.3 Pure Tone Audiometry	11

1.4 Best Procedure for Pure tone audiometry	12

1.5 Advantage and Disadvantage of manual self-testing	12

1.6 Audiogram Interpretation	14

Chapter 2 Purpose of the Project	17

2.2 Goal of the Project	17

2.3 Motivation	17

2.4 Considerations	17

2.5 Measurement	17

Chapter 3 Setting up the development environment	18

3.1 Development tools	18

3.2 Implementing Source Control with Git	18

3.3 Using Source Tree Version Control System and adding git-hub	21

3.4 Android Activities	25

4.5 Android life-cycle	25

Chapter 4 Project Management	27

4.1 User Story Mapping	28

4.2 Pivotal Tracker to track the project backlog	28

4.4 Android Best Practises	30

Chapter 5 Requirements Analysis	31

5.1 Use Case Diagram	31

5.2 Class Diagrams	32

Chapter 6 System Design	34

6.1 UX/UI Design	35

Chapter 7 System Implementation	38

8.1 Implementing the Activities	38

7.2 Implementing Frequency Generator	39

7.3 Implementing Audiogram with Androidplot library	41

7.4 Implementing Fragments	42

7.5 Implementing Rate this app plug in	43

Chapter 8 Testing	44

Chapter 9 Conclusion	45

Appendices	46

Appendix A: Code for FrequencyGen.java	46

Appendix B: User Story Mapping	48

Appendix C: XML code for main_menu.xml	49

Appendix D: Fragments in hearingtest_view.xml	50

1.	Frequency_fragment.xml	51

2.	Audiogram_fragment.xml	52

3.	Java code for FragmentFrequency.java	52

4.	Java code for FragmentAudiogram.java	53

Appendix E: Java code	54

1.	MainMenu.java	54

2.	HearingTestActivity.java	55

3.	MoreInfo.java	61

4.	MoreInfo2.java	62

5.	StartTest.java	62

6.	InfoView1.java	63

7.	InfoView2.java	63

8.	InfoView3.java	64

9.	InfoView4.java	64

Appendix F: Log file of changes	65

Appendix G: The AndroidManifest.xml	67

Appendix H: Strings Resource file	69

Appendix I: Layout Files	71

1.	calibration_info.xml	71

2.	infoview1.xml	72

3.	infoview2.xml	72

4.	infoview3.xml	73

5.	infoview4.xml	73

6.	loadtest.xml	74

7.	loadprev_test.xml	74

Appendix J: Open Source Libraries	74

1.	SimpleXYSeries.java by AndroidPlot developers	74

2.	RateThisApp.java by Keisuke Kobayashi	79

Bibliography	82

Tables

Table 1-1 Grade of impairment	8

Table 2-1 Noise levels during audiometric testing: Accordance with ANSI S3.1-1991.	12

Figures

Figure 1 Diagram of ear.	10

Figure 2 Typical Audiogram	11

Figure 3 Typical activity flow of hearing test	13

Figure 4 the audiogram shows loudness and examples of their environments	14

Figure 5 Presbyacusis audiogram	15

Figure 6 Noise induced audiogram	15

Figure 7 Git- Hub Client.	19

Figure 8 Git-Hub Bash module.	19

Figure 9 Checkout style using UNIX style	20

Figure 10 Initial commit of project to git.	20

Figure 11 Git config commands	23

Figure 12 Pushing updates to repository	24

Figure 13 Sprint cycle	27

Figure 14 Progress report of project	28

Figure 15 Backlog stories and backlog items	29

Figure 16 Gantt chart of project backlog items	30

Figure 17 Use-Case Diagram of main menu	31

Figure 18 Simple class diagram.	32

Figure 19 Class diagram obtained from analysis	33

Figure 20 Detailed class diagram of system	33

Figure 21 a professional audiometer used in clinics	34

Figure 22 the architecture of the apk.	35

Figure 23 Quick layout design created of UI	35

Figure 24 Initial mock design of the main menu	36

Figure 25 Final design of the main menu	37

Figure 26 Diagram of how the methods are called for each button	38

Figure 27 Hierarchy of XYPlot	41

Figure 28 XYPlot template created from XML	41

Figure 29 AlertBox of Rate This App	43

Figure 30 AlertBox before user begins	44

Figure 31 Calibration menu	44

Figure 32 Information screen	44

Figure 33 Testing screen	44

# Acknowledgments

I would like to thank Dr Ray Lynch, my project supervisor who thought me Mobile Applications providing me with a very important understanding of application design and architecture and also helped me and gave me guidance during implementation of the project.

Secondly I would like to thank Paula Kelly for introducing me to Java, providing me with excellent knowledge that has been crucial not only for this project but has helped me have a greater overall understanding of Software Design.

Lastly I would like to thank the School of Electronics and Communications Engineering who gave me a chance to enter this course and to receive study from great lectures.

Also would like to thank my family and friends who encouraged me and supported in my effort to develop this project and to complete my final year.

# Abstract

Hearing is an important ability to human beings in which we perceive sounds. Sound waves occur over a wide spectrum of frequencies. The human ear is sensitive to a frequency band within that spectrum that is expressed in decibels (dB). Frequencies capable of being heard by humans are called audio or sonic. The range is typically considered to be between 20 Hz and 20,000 Hz (Hertz). Frequencies higher than audio are referred to as ultrasonic, while frequencies below audio are referred to as infrasonic. A loss of the ability to hear sound frequencies in the normal range that is specified is called hearing impairment.

# Chapter 1 Introduction

A hearing test provides a means to diagnose an individual’s hearing impairment or deficiencies. ISO 8253-1:2010 is an international standard for PTA (Pure Tone Audiometry). This standard specifies procedures and requirements for PTA air conduction and bone conduction. Pure tone audiometry is the measurement of an individual’s hearing across a range of test frequencies using a standardized test method that specifies the procedure for determining the threshold, the range of test frequencies and  presentation levels, and the way thresholds are presented in a graphical manner and including the symbols user to depict objects. We use PTA to evaluate possible hearing losses and to determine the type of hearing loss that an individual may have.

Table 1-1 Grade of impairment

| Grade of impairment | Audiometric ISO value (500, 1000, 2000, 4000) | Impairment Description |
| --- | --- | --- |
| 0 (No impairment) | 25 dBHL | No or very slight hearing problems |
| 1 (Slight impairment) | 26-40 dBHL | Able to hear and repeat spoken words in normal voice |
| 2 (Moderate impairment | 41-60 dBHL | Able to hear but raised voice |
| 3 (Sever impairment) | 61-80 dBHL | Able to hear some words shouted |
| 4 (Profound impairment) | 81 dBHL | Unable to hear and understand a shouted voice |

The above table shows grade of impairment, level of decibels and the impairment description corresponding to each grade. [7]

Hearing loss can be defined as the amount a person’s hearing level changes as a result of some adverse influence. This means that some structure or function of the ear that is crucial to hearing has been damaged.

There are three main forms of hearing loss; Sensor neural affecting the cochlea, Conductive hearing loss affecting the ear canal and mixed hearing loss which is a combination of both sensor neural and conductive hearing loss.

The World Health Organization (WHO) defines “disabling hearing impairment in adults as a permanent unaided hearing threshold level (average for frequencies 0.5, 1, 2, 4 kHz) for the better ear of 41 dB or greater (WHO, 2001).2 In children under 15 years of age, disabling hearing impairment is defined as permanent unaided hearing threshold level (average for frequencies 0.5, 1, 2, 4 kHz) for the better ear of 31 dB or greater.” [7]

The auditory pathway includes the external ear, the middle ear and the inner ear, followed by the auditory nerve ending up in the auditory centres in the auditory cortex.

- The external ear consists of the pinna, ear canal and eardrum. Sound travels down the ear canal, through the eardrum and causing it to move or vibrate.

- The middle ear is a space behind the eardrum that contains three small bones called ossicles. This string of tiny bones is connected to the eardrum at one end and to the oval window at the other end which connects to the inner ear. Vibrations from the eardrum cause the ossicles to vibrate which, in turn, creates movement of the fluid in the inner ear.

- Movement of the fluid in the inner ear, or cochlea, causes changes in hair cells. This movement of the hair cells sends electric signals from the inner ear up the auditory nerve to the brain. [7]

The brain then interprets these electrical signals as sound. Figure 1 - shows the different areas of the ear described below. [11]

Figure 1 Diagram of ear.

From the above diagram we can see the middle ear this is where most conductive hearing loss occurs this is treated by medical treatment or through surgery. Sensor neural hearing loss occurs in the inner ear as the nerves become damaged and do not properly transmit their signals to the brain. This happens become of aging of the inner ear cells. Sensor neural hearing loss is the most common type hearing loss among adults. It is not often medically or surgically treatable. This type of hearing loss is can be treated with the help of hearing aids. When hearing loss is conductive and Sensor neural this is called mixed hearing loss.

## 1.1 How we hear sound?

Sound waves enter the outer ear and through the ear canal. The eardrum vibrates from incoming sound waves and sends those vibrations to bones in middle malleus, incus and stapes. The sound vibrations go from the air to fluid vibrations in cochlea of the inner ear. Once fluid inside cochlea vibrates waves form along the basilar membrane. When hair cells move, the bending of the hair cells causes chemicals to create an electrical signal. The auditory nerve than carries this signal to the brain which then is translated into a sound that we hear and understand. [15]

## 1.2 The Audiogram

The audiogram plots decibel (dB) values on the Y-Axis on an audiogram. If a loss of hearing is present then the graph will have higher points with increased dB signal where the person is having difficulty hearing the tone. The more severe the hearing loss the more the dB values on the graph will going downwards. On the X-Axis we have the plotting of the frequency pure tones in hertz (Hz). An Audiogram usually plots sound on the left lowest 125Hz and increasing to 8000 KHz.

Figure 2 Typical Audiogram

## 1.3 Pure Tone Audiometry

Hearing thresholds of humans can be found by using behavioural hearing tests or physiological tests. An audiogram can be obtained using a behavioural hearing test called Audiometry. For humans the test involves different tones being played at a specific frequencies (pitch) and intensity (loudness). When this test is done with a doctor, the person hears the sound they raise their hand or press a button so that the tester knows that they have heard the sound. The lowest sound the person hears is recorded. The entire auditory system from ear canal to auditory cortex is tested. When a test is decided a method of selecting the correct method has to be chosen. There is three main methods manual, self-recording and microprocessor. I decided to choose manual audiometry method for the design of the application as it is most practical as the user decides the test environment. However this does not make the application design the most scientifically accurate. In the manual audiometry the frequency and intensity are controlled by the user. When hearing is tested the frequencies of the range of 125Hz to 8000 Hz are usually tested.  One of the most important things in audiometry is the environment in which the subject is undertaking the test. There are different standards depending on the region which require different calibration.

Table 2-1 Noise levels during audiometric testing: Accordance with ANSI S3.1-1991.

| Octave-Band Centre Frequency | 500 Hz | 1000 Hz | 2000 Hz | 4000 Hz | 8000 Hz |
| --- | --- | --- | --- | --- | --- |
| ANSI S3.1-1991 (Rounded to the nearest whole decibel) | 22 | 30 | 34 | 42 | 45 |
| OSHA Table D-2 | 27 | 30 | 32 | 42 | 45 |
| OSHA Table D-1 | 49 | 40 | 47 | 57 | 62 |

## 1.4 Best Procedure for Pure tone audiometry

When beginning a test it is best to start with the right ear but the user decides what ear he wants checked. A pulsing tone that goes on and off is usually recommended; in the design of the application the user presses a button to hear the tone. The optimal test environment is quiet and free from anything that could distract user.  Sometimes it is useful to wear noise-reducing earphones for best performance but regular earphones can also be used.

## 1.5 Advantage and Disadvantage of manual self-testing

The advantage of self-testing is that it can be done regularly and quickly as no specialised equipment is required. On the other hand the disadvantage is that with manual testing a user can make mistakes, press wrong button or test same ear twice. This can introduce errors into the audiogram which can make it useless from a scientific view. There are many things that can go wrong with accurate results example; too much background noise, earphones not placed properly and tester bias when a person cannot hear.

Figure 3 Typical activity flow of hearing test

From the above a typical hearing test consists of the following choices. A user selects if we can hear tone or cannot as the option is selected the result is recorded on a graph, in this case an Audiogram. Audiograms help us to explain audiometric readings.

## 1.6 Audiogram Interpretation

In order to interpret audiogram there are usually certain characteristics that explain the results of the audiogram. These characteristics can determine the type of hearing loss.

Below are some examples of audiograms and sample interpretation.

- Monaural loss: Loss is only in one ear.

- Binaural loss: Loss is in both ears.

- Symmetrical: Hearing is relatively uniform in both ears.

- Asymmetrical: Hearing loss in one ear is significantly worse than the other ear.

The shape of the audiogram is also an important factor to determining the type of loss. There is flat loss, sloping loss and other shapes show different aging and noise damage on the audiogram.

Figure 4 the audiogram shows loudness and examples of their environments

The above is a good example of the different scenarios and their associated decibel levels that a person can imagine. With a low sound level at 10 dBs the chirping of birds and at 120 dBs a pneumatic drill and a jet aircraft.  [20]

Presbyacusis is an age related hearing loss. Affects high frequencies much more than lower. From the above we can see that sounds have to be made louder before the person can hear them.

Figure 5 Presbyacusis audiogram

Noise induced hearing loss is where a hearing organ has been damaged for example the cochlea.

Figure 6 Noise induced audiogram

Symmetrical hearing loss is when both ears have same hearing loss.

# Chapter 2 Purpose of the Project

The purpose of this project was to develop an application that would help identify common hearing impairment by using an audiogram which would picture how a person hears. The audiogram is used to describe the hearing of a person across different frequencies. It can be used as a tool to determine amount of damage done or determine the cause.

## 2.2 Goal of the Project

The goal was to develop and application to determine hearing impairments in adults.

## 2.3 Motivation

An easy and simple application that will allow users to quickly do hearing tests for an assessment, where they get results by reading the audiogram.

## 2.4 Considerations

The problem of hearing impairment is an increasing problem across most populations of the world and is something that people should be aware of and by taking hearing tests they can be made more aware of their own level of hearing and how to best keep it healthy. In the EU it is estimated that more than 55 million people are having hearing impairments and the costs in the EU is estimated to 160 billion Euros per year [10]. According to a study, a mild hearing loss costs 2000 euros per individual each year, a moderate costs 6,600 and severe 11,000 euros.[9]

From the above studies we can see that it is a problem that needs to be carefully analysed and more research needs to be done.

## 2.5 Measurement

The advantage brought to users through this application is that it provides free and quick self-test ability. Where-as many Android applications or web applications require connection to the internet using databases, this current project does all processing locally on the users device.

# Chapter 3 Setting up the development environment

## 3.1 Development tools

The integrated development environment (IDE) to be used will be Eclipse Juno, which is used to develop Android Applications using Java to develop the application. For the project management aspect of the project MS Visio will be used. To comply with ISO 8253 -1:2010 open source graph plotting library called AndroidPlot to generate the audiogram will be used. [5].

There are many different versions of Eclipse and a newer Android Studio was released, however Eclipse Juno will be used as it is the same as Android Studio which mainly has a graphical and performance upgrade for newer PC’s. However a switch to Android Studio is simple as all that is required is importing the project into Android Studio.

## 3.2 Implementing Source Control with Git

Version control is one of the most important aspects of a software project. It’s an essential part of development where we can process the management of several files and different revisions of those files. The version control system provides us with a central hub where the code is located, it maintains a history of the work on the project, central place for backup and making separate copies of the project is not required and it also allows reversibility, meaning we can reverse changes [12]. For this project I learned to use Git-Hub where I created my own repository and licensed my project under the Apache license. In order to implement source control into the project a number of steps had to be taken. First the installation of the git-hub client for windows was required.

Figure 7 Git- Hub Client.

Figure 8 Git-Hub Bash module.

I chose the Bash module as I am familiar with Bash scripting and chose the associated extensions for git files.

Figure 9 Checkout style using UNIX style

Figure 10 Initial commit of project to git.

The repository is created along with apache license.

## 3.3 Using Source Tree Version Control System and adding git-hub

I used Source Tree by Atlassian for version control which is a very powerful VCS. These stores all the project files onto Git including source code files, documentation, build files configuration files, assets and libraries.

[16]

Once I have installed source tree, I linked the git hub account.

The project is added once we create a copy of it within the folder created for Git-Hub.

We add some comments to HearingTestActivity class.

Any changes made to code are saved in source tree and we can see from the above that highlighted code in green is what we added.

Once happy with changes made we can commit changes made.

To commit changes we have to install Git for windows this adds the command line tools to run git.

Figure 11 Git config commands

Once I set the git config commands in git bash shell we can commit any changes made in source tree.

Figure 12 Pushing updates to repository

The files are then added to the main git repository.

## 3.4 Android Activities

An activity represents a single screen that the user sees on the device. An application usually consists of multiple activities. Activities are the most observable part of the application. In Android, you can be looking at an activity of one application, but shortly after you could start another activity in a completely separate application. For example, if you are in the Calendar application and you decide to call a friend, you would be launching the activity to bring up the phone application in the Calendar application.

## 4.5 Android life-cycle

Activity can be expensive on the device CPU and RAM. It can sometime involve creating a new Linux process, allocating memory for all the new objects, inflating the objects from XML layouts, and setting up the screen. In Android the activity life cycle is managed by the Activity Manager.

Activity Manager is responsible for creating, destroying, and managing activities. For example, when the user starts an application for the first time, the Activity Manager will create its activity and put it onto the screen. Later, when the user switches screens, the Activity Manager will move that previous activity to a holding place. This way, if the user wants to go back to an older activity, it can be started more quickly. Older activities that the user hasn’t used in a while will be destroyed in order to free more space for the currently active one. This mechanism is designed to help improve the speed of the user interface and thus improve the overall user experience.

Programming for Android is conceptually different than programming for some other environments. In Android, you find yourself responding more to certain changes in the state of your application rather than driving that change yourself. It is a managed, container-based environment similar to programming for Java applets or servlets. So, when it comes to an activity life cycle, you don’t get to say what state the activity is in, but you have plenty of opportunity to say what happens during the transitions from state to state. The figure below shows the states that an activity can go through. [13]

Managing the lifecycle of activities by implementing call-back methods is crucial to developing a strong application. The lifecycle of an activity is directly affected by its association with other activities,

Call-back methods are essentially in the three states as follows:

- Resumed

The activity is in the foreground of the screen and has user focus.

- Paused

Another activity is in the foreground and has focus, but this one is still visible. That is, another activity is visible on top of this one and that activity partially transparent,

- Stopped

The activity is completely hidden by another activity.

# Chapter 4 Project Management

In this project I chose an iterative approach towards development. With the UX design, UI design and software development stages all iterative pieces of each sprint cycle. Bit by bit each feature was added. With an iterative approach I was able to focus on each small piece and create a working bit. My aim was to have a working piece of software by every week; this further enabled clarity and gave me quick feedback by demonstrating different version to peers or friends.

Figure 13 Sprint cycle

Each piece of the development cycle was broken down into 1 to 2 weekly sprints which consisted of adding functionality to each part of the project.  I used user story mapping which helped draw out the project, this acted as a visual guide to the project, which solidified my understanding of the requirements and if any changes t occurred were easy foreseeable and changeable.

## 4.1 User Story Mapping

User story mapping it a practise in that’s helps us create a visual structure of release planning for each item in the backlog. The activities in the map are arranged in order of priority. The story map creates version of the software that we can visualise and see what parts will affect other parts. This approach helps with incremental delivery and creates deadlines. Refer to image of Appendix B.

User story is description in one or two sentences as to what the end user of the system wants; it’s an effort to capture requirements. In practise it is done on simple sticky notes.

## 4.2 Pivotal Tracker to track the project backlog

The main project management software I used for this project was pivotal tracker, this is a very powerful tool that helps with task management, velocity tracking, iteration planning and also produces progress charts. [18]

Figure 14 Progress report of project

The above shows some of the main features I added for each sprint. Continually recording results and keeping track of deliverables required dedication and patience towards project management. This further helped me establish a clear and focused view of the requirements.

Figure 15 Backlog stories and backlog items

The backlog stories are converted into working software after 1-2 weeks of development they are then added as features at each stage of the product.  Iterations enable us to adjust to changes when they are necessary. As the change of requirements in software is inevitable these stories are not concrete as they change depending upon what is working and what does not.

The benefits of this agile iteration approach are powerful because as time is the most valuable asset, for this project I had to carefully manage my time by keeping track of each task.

Figure 16 Gantt chart of project backlog items

## 4.4 Android Best Practises

Some of the best practises that I followed in developing for Android were as follows:

- App installs on the users external disk. Refer to Appendix E.

- Features are easily accessible for different type of users from young to old.

- Layout of information is kept in uniform order and with consistent look and feel to the overall application design.

- Responsive and stable programming to avoid crashes by always catching exceptions.

- A change log of each major and minor change was maintained for each cycle. Refer to Appendix F.

- Maintained the strings resources file, which contains text strings so that we do not have to add the strings into the main body of the code, but instead we reference each string that we want to use from the strings.xml file. This is crucial practise that enables clean code and also provides us with the ability to localize our app into different languages with a much easier technique. Refer to Appendix H.

- High res and low res icons for icons and images.

# Chapter 5 Requirements Analysis

A requirements document specifies the purpose of the system. With the Hearing Test Application the users must be able to test their left and right ear, view results from these tests, save, load tests and view general information about hearing.

## 5.1 Use Case Diagram

The user interface has a screen that displays menu to the user and a simple interface so user can navigate.

Figure 17 Use-Case Diagram of main menu

The use-case diagram helps model the user’s interactions between the application and the user.

The software encapsulates the functionality and tries to mimic how a hardware device would be. In the design of this application I used a class for frequency generation and a XY plotter to act as the audiogram.

## 5.2 Class Diagrams

Figure 18 Simple class diagram.

The above class demonstrates the three main classes that will be used. Each represents an association with a “has-a” relationship which demonstrates composition. Composition allows us to place sub-objects inside another class. Composition is a form of code reuse so it is useful to include this in the design. This means Hearing Test class has a frequency generator and “has-a” audiogram. This means that the hearing test class contains objects of type frequency generator and audiogram fragment.

Classes used in the system:

- Hearing Test Activity.java

- Frequency generator.java

- Frequency Fragment.java

- AudiogramFragment.java

- MainMenu.java

- MoreInfo.java

Figure 19 Class diagram obtained from analysis

Figure 20 Detailed class diagram of system

Detailed class diagram represents most of major classes that user interacts with.

# Chapter 6 System Design

Figure 21 a professional audiometer used in clinics

The above is a professional audiometer used in clinics it is priced at around €1650. [14]

The classes can be modelled by observing the mechanism of the audiometer. It contains a DSP unit which produces the pure tone so we create a frequency generator class, the audiometer may produce an audiogram with separate software on a pc but we can create a class that will either hold the information for audiogram or render it onto the users screen. The main activity is the Hearing Test Activity, this class contains main behaviour of the overall system, and this is where user selects which ear to test and which frequency to test at.

The diagram below demonstrates the architecture of the android package file which consists of the classes that are in the Dalvik Executable format (DEX).  Dalvik is virtual machine in Android operating system. Programs are written in Java and compiled into byte code for JVM which is then translated into Dalvik byte code and stored in .dex or .odex (Optimized Dalvik).

Figure 22 the architecture of the apk.

## 6.1 UX/UI Design

Figure 23 Quick layout design created of UI

Using pencil UI designer we make quick designs, to visualise the look and feel of the application, which later can be adapted into real design in the layout of the XML file. [21]

Quick UI wireframe models can sometimes be quickly created by hand but sometimes it is more useful to create diagrams with tools such as pencil.

Figure 24 Initial mock design of the main menu

Figure 25 Final design of the main menu

I revised the design into a more elegant and simple view with clickable image buttons. Refer to XML layout in Appendix D.

# Chapter 7 System Implementation

## 8.1 Implementing the Activities

Figure 26 Diagram of how the methods are called for each button

Each button has as onClick() method which the is specified in the XML file. Once button is clicked, an intent object is created which takes instance of current class and class to be launched. Intents provide the ability to bind classes together which helps us to launch them along. Refer to Appendix E.

Another important aspect to activities is declaring them in the Android Manifest because if not declared the application will crash with an exception error.  In the Manifest we use an activity tag and pass in location of class in the project package. Refer to Appendix G.

## 7.2 Implementing Frequency Generator

xt=A Sin 2π n fafs

Generating the pure tone requires the use of basic sine wave.  In the Frequency Generator class we create an array of generated sound samples and pass it into the AudioTrack.write function. This function takes data to the audio sink for playback in streaming mode. The byte class I sued as it wraps primitive value byte in an object. The encoding scheme is PCM 16 bits per sample. Pulse code modulation encodes an audio waveform in the time domain. The frequency and sample rate measures how many samples are plated each second. We use getChannelConfiguration which return the configured channel configuration. [23]

The encoding plays an important role as the byte array is in played in form of audio. The audio track write method takes in three parameters one is the byte array this is the audio data, second is the offset bit and last is the size of the bytes.

We create an object of type short which is a 16-bit signed two’s complement integer. The short class wraps a value of primitive type in an object. Refer to Appendix A.

Configuring the sample rate 44,100 gives the highest quality sound. We test each pure tone and compare if it sounds is as good [24].

## 7.3 Implementing Audiogram with Androidplot library

In order to render the audiogram we require a comprehensive library to create our audiogram. There are two most popular open source libraries available, both were tried and tested, these included AndroidPlot and GraphView. [24]

After testing both GraphView and AndroidPlot, the latter was more useful as it contained helpful examples and its source code was more understandable. To implement AndroidPlot within the project first we have to include it within the project build path. Refer to Appendix D: Audiogram_fragment.xml

The main audiogram is implemented into the HearingTestActivity class. This class contains a method to render XYPlot which we associate with out XYPlot which we declared in our audiogram XML file.  We create a reference object of type XYPlot this contains information for the plot XY axis and colour of background. This XYPlot class further inherits from XYSeries, XYSeriesFormatter and XYSeriesRenderer. These are all interfaces or abstract classes which provide impartial or no implementation.

Figure 27 Hierarchy of XYPlot

Figure 28 XYPlot template created from XML

## 7.4 Implementing Fragments

Fragments in android can be described as an activity within an activity. I chose to implement fragments as I needed a way to combine multiple activities that required different classes acting on screen view. In my design two fragments were added to one layout view that is the HearingTestActivity. In order to implement this I first created a layout view for the playtone_fragment this is where the user controls the testing options. The other fragment is then of the audiogram which only draws a XYPlot using the Android plot library. The two fragments are then combined into one layout, the reason for doing this is that we can separate the logic of both classes to their own respected layouts and that once we embed them as fragments, the code is less coupled and not totally dependent on other methods .

The implementation of the fragments was one of the trickiest aspects of the project as it took me a long time to implement. There many of ways to implement fragments a there is no best solution so it was trial and error to discover which method worked best. The method that worked for me was where I had fragments tags declared in the hearing test layout and have a class tag defined and also a tools tag for each fragment. The tools tag defines which layout is rendered and is very important. Refer to code in Appendix D.

Figure 30 Diagram of fragment associations.

To complete the implementation of fragments we create two classes for each xml file. These two classes contain an onCreateView method which inflates the layout into the hearingtest_view.xml activity. The inflater instantiates a layout XML file into the View object.  The inflation uses XML file at build time. Last thing to make sure is to include the android-support-v4.jar file in the project build path. Refer to Appendix D.

## 7.5 Implementing Rate this app plug in

I implemented a simple rate this app plugin written by Keisuke Kobayashi. It is an open source library and was available through git-hub. I decided to implement this feature to add to the applications user experience. As this plugin provides the user with a simple prompt alert box which provides a layer of interactivity to rate the application on the Google Play Store.

Implementing the library is very simple, RTA plug in is very useful in that provides an intuitive and simple way to receive user feedback. It encourages user interactivity by prompting user to give feedback and rate the app depending on their experience.

Figure 29 AlertBox of Rate This App

We include a small method in our MainMenu which is an onStart method. This is called right after the onCreate method creates the view.  When the view is visible to the user the method runs two methods belonging to the RateThisApp class. The first onStart method checks user install date and number of times person has launched application, based upon these parameters it then shows the showRateDialog prompting the user to rate the app in the Google Play Store.

Figure 30 AlertBox before user begins

Figure 31 Calibration menu

Chapter 8 Testing

Figure 32 Information screen

Figure 33 Testing screen

# Chapter 9 Conclusion

In conclusion I learned how to create an Android application with good design. I learned how to use version control system and effectively use it within a project and with Git-Hub.  If given more time, the application could be fully completed with a results screen that shows user a breakdown of what type of hearing loss they may have. A database can also be created using XML to store the user’s test results and previous tests. I have licensed the project under the apache license 2.0 and it is available on Git-Hub, making it open source to the developer community but attribution has to be made. I aim to complete the application and give it a more professional look and feel with fully working futures and to make it the best free hearing test application on the Android App Store. A lot of time was spent on initially learning how to create Android applications with good design through trial and error, learning to integrate AndroidPlot library and fragments all took a lot of time to get working. It was easy to underestimate the project because hearing tests seem simple in concept but gradually throughout the development it was overall a great learning but at the same time a surprising challenge.

# Appendices

## Appendix A: Code for FrequencyGen.java

/* * {This class is used to run methods based on what user chooses * for the Frequency Generator } * * @version Build {1.0} (14 May 2015) * @author Junaid Malik */ package com.example.testtonegenv10; import android.app.Activity; import android.media.AudioFormat; import android.media.AudioManager; import android.media.AudioTrack; import android.media.MediaPlayer; import android.widget.Toast; // this class is based on Marble Mice: Generate And Play A Tone In Android // www.marblemice.blogspot.ie/2010/04/generate-and-play-tone-in-android.html public class FrequencyGenerator extends Activity { public final int duration = 5; // length of tone in seconds public int sampleRate = 44100; // sample rate at 44,100 times per second public final int numSamples = duration * sampleRate;// in order to avoid // distortion // number of samples = // duration multiplied // by sample rate public final double sample[] = new double[numSamples]; public double freqOfTone; // frequency in Hz public final byte generatedSound[] = new byte[2 * numSamples]; // array of // genSoun = // 2 times // the // number of // samples public static final int LEFT_EAR = 1; // immutable int for left ear public static final int RIGHT_EAR = 2; // immutable int for light ear public AudioTrack audioTrack = null; // null object of type audioTrack public void genTone() { // fill out the array with samples for (int counter = 0; counter < numSamples; ++counter) { sample[counter] = Math.sin(2 * Math.PI * counter // x(t)=A Sin 2pi n // fa/fs / (sampleRate / freqOfTone)); } // convert to 16 bit pcm sound array // assumes the sample buffer is normalised. int index = 0; // enhanced for loop going through sampa array for (final double doubleValue : sample) { // scale to maximum amplitude of 32,767 final short value = (short) ((doubleValue * 32767)); // in 16 bit wav PCM, first byte is the low order byte // 0xff00 is hex literal for the value 65,280 bitwise // it resets the rightmost 8 bits // operation, right shift to 8bits // 0x00ff only the rightmost 8 bits are kept // value is anded with 0x00FF // Anding with 8 zeros we are masking off 8 right MSB generatedSound[index++] = (byte) (value & 0x00ff); generatedSound[index++] = (byte) ((value & 0xff00) >>> 8); } } /* * audiotrack plays pcm audio buffers data is pushed to audiotrack object in * streaming mode the system writes continous data using write() method this * method takes three params audioData, offfsetinBYtes and sizeinBytes */ public void playSound() { audioTrack = new AudioTrack(AudioManager.STREAM_MUSIC, sampleRate, AudioFormat.CHANNEL_OUT_STEREO, AudioFormat.ENCODING_PCM_16BIT, numSamples, AudioTrack.MODE_STATIC); audioTrack.write(generatedSound, 0, generatedSound.length); audioTrack.getChannelConfiguration(); try { audioTrack.play(); } catch (Exception e) { // error message if not playable Toast.makeText(getApplicationContext(), "Error playing audio", Toast.LENGTH_SHORT).show(); } } public void playTone() { playTone(1.0f, 1.0f); // this method plays tone with full // volume in both ear } public void playTone(float left, float right) { // overloaded playTone // method with volume // determined by user Float leftVolume = left; Float rightVolume = right; try { audioTrack.write(generatedSound, 0, generatedSound.length);// pass // genSound // into // write // method audioTrack.setStereoVolume(leftVolume, rightVolume); // left and // right // are set // by // user // choice audioTrack.play(); } catch (Exception e) { // error message if not playable Toast.makeText(getApplicationContext(), "Error playing audio", Toast.LENGTH_SHORT).show(); } } public void playTone(int earSelect) { switch (earSelect) { case LEFT_EAR: playTone(1.0f, 0.0f); break; case RIGHT_EAR: playTone(0.0f, 0.05f); break; } } public void start() { audioTrack.play(); } public void stop() { audioTrack.stop(); } }

## Appendix B: User Story Mapping

## Appendix C: XML code for main_menu.xml

<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"

xmlns:tools="http://schemas.android.com/tools"

android:layout_width="match_parent"

android:layout_height="match_parent"

android:background="#FFFFFF"

android:paddingBottom="@dimen/activity_vertical_margin"

android:paddingLeft="@dimen/activity_horizontal_margin"

android:paddingRight="@dimen/activity_horizontal_margin"

android:paddingTop="@dimen/activity_vertical_margin"

tools:context="com.example.testtonegenv10.MainMenu" >

<RelativeLayout

android:id="@+id/relativeLayout1"

android:layout_width="match_parent"

android:layout_height="match_parent" >

<TextView

android:id="@+id/textView2"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentLeft="true"

android:layout_below="@+id/imageButton1"

android:layout_marginLeft="30dp"

android:text="Begin test" />

<ImageButton

android:id="@+id/imageButton1"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignLeft="@+id/textView2"

android:layout_alignTop="@+id/imageButton2"

android:background="#FFFFFF"

android:onClick="beginTest"

android:src="@drawable/blue_ear" />

<TextView

android:id="@+id/TextView01"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignBaseline="@+id/textView2"

android:layout_alignBottom="@+id/textView2"

android:layout_alignLeft="@+id/imageButton2"

android:text="Load test" />

<ImageButton

android:id="@+id/imageButton3"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_above="@+id/TextView02"

android:layout_alignLeft="@+id/TextView01"

android:background="#FFFFFF"

android:onClick="moreInfo"

android:src="@drawable/about_icon" />

<TextView

android:id="@+id/TextView02"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignLeft="@+id/imageButton3"

android:layout_alignParentBottom="true"

android:layout_alignRight="@+id/imageButton3"

android:layout_marginBottom="61dp"

android:text="    About" />

<ImageButton

android:id="@+id/imageButton4"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_above="@+id/TextView02"

android:layout_alignLeft="@+id/textView2"

android:background="#FFFFFF"

android:onClick="moreInfo2"

android:src="@drawable/how_hearing_works" />

<TextView

android:id="@+id/textView3"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignBaseline="@+id/TextView02"

android:layout_alignBottom="@+id/TextView02"

android:layout_alignLeft="@+id/imageButton4"

android:text="Learn more?" />

<TextView

android:id="@+id/textView1"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentTop="true"

android:layout_centerHorizontal="true"

android:gravity="top|center"

android:text="@string/hello_world"

android:textSize="25dp" />

<ImageButton

android:id="@+id/imageButton2"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignRight="@+id/textView1"

android:layout_below="@+id/textView1"

android:layout_marginTop="76dp"

android:background="#FFFFFF"

android:onClick="loadTest"

android:src="@drawable/load2" />

</RelativeLayout>

</LinearLayout>

## Appendix D: Fragments in hearingtest_view.xml

<?xml version="1.0" encoding="utf-8"?>

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"

xmlns:tools="http://schemas.android.com/tools"

android:layout_width="fill_parent"

android:layout_height="fill_parent"

android:orientation="horizontal" >

<fragment

android:id="@+id/audiogram_fragment"

android:layout_width="match_parent"

android:layout_height="match_parent"

android:layout_alignParentBottom="true"

android:layout_marginBottom="140dp"

class="com.example.testtonegenv10.FragmentAudiogram"

tools:layout="@layout/audiogram_fragment" />

<fragment

android:id="@+id/playtone_fragment"

android:layout_width="match_parent"

android:layout_height="149dp"

android:layout_alignParentBottom="true"

android:layout_alignParentLeft="true"

class="com.example.testtonegenv10.FragmentPlayTone"

tools:layout="@layout/playtone_fragment" />

</RelativeLayout>

### Frequency_fragment.xml

<?xml version="1.0" encoding="utf-8"?>

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"

xmlns:tools="http://schemas.android.com/tools"

android:layout_width="match_parent"

android:layout_height="match_parent"

android:orientation="vertical"

tools:context="com.example.testtonegenv10.FragmentPlayTone" >

<Button

android:id="@+id/cannotHearButton"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentBottom="true"

android:layout_alignParentRight="true"

android:text="Cant hear" />

<Button

android:id="@+id/canHearButton"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentBottom="true"

android:text="Can hear" />

<Button

android:id="@+id/finishButton"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentBottom="true"

android:layout_toLeftOf="@+id/cannotHearButton"

android:text="Barely hear" />

<RadioGroup

android:id="@+id/radioGroup1"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_above="@+id/canHearButton"

android:layout_alignParentLeft="true"

android:orientation="horizontal" >

<RadioButton

android:id="@+id/leftEarButton"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:checked="true"

android:text="Left" />

<RadioButton

android:id="@+id/rightEarButton"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:text="Right" />

</RadioGroup>

<Button

android:id="@+id/playFrequencyButton"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignBottom="@+id/radioGroup1"

android:layout_alignParentRight="true"

android:text="Play Tone" />

<Button

android:id="@+id/nextFrequencyButton"

style="?android:attr/buttonStyleSmall"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_above="@+id/playFrequencyButton"

android:layout_alignParentRight="true"

android:text="Next Frequency" />

</RelativeLayout>

### Audiogram_fragment.xml

<?xml version="1.0" encoding="utf-8"?>

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"

xmlns:tools="http://schemas.android.com/tools"

android:layout_width="match_parent"

android:layout_height="match_parent"

android:orientation="vertical"

tools:context="com.example.testtonegenv10.FragmentAudiogram" >

<TextView

android:id="@+id/textView1"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentLeft="true"

android:layout_alignParentRight="true"

android:layout_alignParentTop="true"

android:background="#E0FFF5"

android:text="You can select options to test your hearing"

android:textSize="16sp" />

<com.androidplot.xy.XYPlot

android:id="@+id/mySimpleXYPlot"

android:layout_width="313dp"

android:layout_height="210dp"

android:layout_alignParentBottom="true"

android:layout_alignParentLeft="true"

android:layout_alignParentRight="true"

android:layout_below="@+id/textView1"

android:layout_weight="3.49" />

</RelativeLayout>

### Java code for FragmentFrequency.java

package com.example.testtonegenv10;

import android.os.Bundle;

import android.support.v4.app.Fragment;

import android.view.LayoutInflater;

import android.view.View;

import android.view.ViewGroup;

public class FragmentFrequency extends Fragment {

public FragmentFrequency() {

} // empty constructor

@Override

public View onCreateView(LayoutInflater inflater, ViewGroup container,

Bundle savedInstanceState) {

// ---Inflate the layout for this fragment---

return inflater.inflate(R.layout.frequency_fragment, container, false);

}

}

### Java code for FragmentAudiogram.java

package com.example.testtonegenv10;

import android.os.Bundle;

import android.support.v4.app.Fragment;

import android.view.LayoutInflater;

import android.view.View;

import android.view.ViewGroup;

public class FragmentAudiogram extends Fragment {

public FragmentAudiogram() {

}

public View onCreateView(LayoutInflater inflater, ViewGroup container,

Bundle savedInstanceState) {

return inflater.inflate(R.layout.audiogram_fragment, container, false);

}

}

## Appendix E: Java code

### MainMenu.java

/*

* {This class is used to run methods based on what user chooses

*  for the main menu }

*

* @version Build {0.6} (26 March 2015)

* @author Junaid Malik

*/

package com.example.testtonegenv10;

import com.kskkbys.rate.RateThisApp;

import android.app.Activity;

import android.content.Intent;

import android.os.Bundle;

import android.view.View;

public class MainMenu extends Activity {

@Override

public void onCreate(Bundle savedInstanceState) { // activity is strarted

// here

super.onCreate(savedInstanceState);

setContentView(R.layout.main_menu); // main menu view is loaded first

}

public void beginTest(View v) {

Intent intent = new Intent(this, StartTest.class); // intent used to launch activity

startActivity(intent);

}

public void moreInfo(View v) {

Intent intent = new Intent(MainMenu.this, MoreInfo.class);

startActivity(intent);

}

public void loadTest(View v) {

Intent intent = new Intent(MainMenu.this, LoadTestActivity.class);

startActivity(intent);

}

public void moreInfo2(View v) {

Intent intent = new Intent(MainMenu.this, MoreInfo2.class);

startActivity(intent);

}

// code from rate this app by Keisuke Kobayashi

@Override

protected void onStart() {

super.onStart();

// Monitor launch times and interval from installation

RateThisApp.onStart(this);

// If the criteria is satisfied, "Rate this app" dialog will be shown

RateThisApp.showRateDialogIfNeeded(this);

}

}

### HearingTestActivity.java

package com.example.testtonegenv10;

import java.util.Arrays;

import java.util.Iterator;

import android.app.AlertDialog;

import android.content.Context;

import android.content.DialogInterface;

import android.graphics.Bitmap;

import android.graphics.BitmapFactory;

import android.graphics.BitmapShader;

import android.graphics.Color;

import android.graphics.Matrix;

import android.graphics.RectF;

import android.graphics.Shader;

import android.media.AudioManager;

import android.os.Bundle;

import android.util.Log;

import android.view.KeyEvent;

import android.view.View;

import android.view.View.OnClickListener;

import android.widget.Button;

import android.widget.RadioButton;

import android.widget.Toast;

import com.androidplot.xy.BoundaryMode;

import com.androidplot.xy.LineAndPointFormatter;

import com.androidplot.xy.PointLabelFormatter;

import com.androidplot.xy.XYPlot;

import com.androidplot.xy.XYSeries;

import com.androidplot.xy.XYStepMode;

public class HearingTestActivity extends

android.support.v4.app.FragmentActivity implements OnClickListener {

public HearingTestActivity() { // empty default constructor

}

private Button canHearButton, cannotHearButton, nextFrequencyButton,

finishButton, playFrequencyButton;

private RadioButton rightEarButton, leftEarButton;

static  int defaultdB = 40; // default dB level (yVal point)

private FrequencyGenerator frequencygen = new FrequencyGenerator(); // create FreqGen

// reference

private FragmentAudiogram audiogram = new FragmentAudiogram(); // create

// AudioGram

// reference

// private AudioManager audioManager; // reference to AudioManager class

private XYPlot mySimpleXYPlot; // reference to XYPlot class

private SimpleXYSeries series1, series2;

//private int[] frequencies;

private int currentFreq = 0;

private int indexYval = 0; // initial index position for yVals

private int indexXvals = 0;

private int FREQ_LEN = 120; // length of array for x vals

private int DB_LEN = 120; // length of array for y vals

// private int[] frequencies = new frequencies;

Integer[] frequencies = {250, 500, 1000, 2000, 4000, 6000, 8000};

Integer[] xVals = new Integer[FREQ_LEN]; // array of type Integer with

// length defined

Integer[] yVals = new Integer[DB_LEN];

@Override

public void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

setContentView(R.layout.hearingtest_view); // setting the view with

// defined XML file

startUp(); // opens up info box

this.setVolumeControlStream(AudioManager.STREAM_MUSIC); // reference to

// current

// volume stream

//frequencies = getResources().getIntArray(R.array.frequencies);

// frequencies = getResources().getIntArray(R.array.frequencies);

leftEarButton = (RadioButton) findViewById(R.id.leftEarButton);

leftEarButton.setOnClickListener(this);// if button clicked

// onClickListener runs

// corresponding onClick()

// method

rightEarButton = (RadioButton) findViewById(R.id.rightEarButton);

rightEarButton.setOnClickListener(this);// if button clicked

// onClickListener runs

// corresponding onClick()

// method

playFrequencyButton = (Button) findViewById(R.id.playFrequencyButton);

playFrequencyButton.setOnClickListener(this);// if button clicked

// onClickListener

// runs corresponding onClick()

// method

nextFrequencyButton = (Button) findViewById(R.id.nextFrequencyButton);

nextFrequencyButton.setOnClickListener(this);// if button clicked

// onClickListener

// runs corresponding onClick()

// method

canHearButton = (Button) findViewById(R.id.canHearButton);

canHearButton.setOnClickListener(this);// if button clicked

// onClickListener runs

// corresponding onClick()

// method

cannotHearButton = (Button) findViewById(R.id.cannotHearButton);

cannotHearButton.setOnClickListener(this);// if button clicked

// onClickListener runs

// corresponding onClick()

// method

// initialize our XYPlot reference:

mySimpleXYPlot = (XYPlot) findViewById(R.id.mySimpleXYPlot);

graphSettings(); // retrieve layout for audiogram

}

// onClick method gets called each time a button is pressed

@Override

public void onClick(View v) {

// audio manager provides access to volume control

// Context is an interface to global info about app environment

// context implementation provided by OS.

AudioManager audioManager = (AudioManager) getSystemService(Context.AUDIO_SERVICE);

// Run the function findViewById and pass it R.id.button1 as parameters

// Find out which button was pushed based on its ID

// Switch statement checks for which button was checked and changes that

switch (v.getId()) {

case R.id.nextFrequencyButton: {

setFreqIndex();

//setFreqValue();

indexYval += 1;

currentFreq += 1;

yVals[ indexYval ] = 40; // set next index to default value 40

updatePlot();

}

case R.id.playFrequencyButton: {

frequencygen.freqOfTone = frequencies[currentFreq];

frequencygen.genTone();

frequencygen.playSound();

}

case R.id.leftEarButton: {

try {

if (leftEarButton.isChecked())

frequencygen.playTone(FrequencyGenerator.LEFT_EAR);

else

frequencygen.playTone(0.0f, 1.0f);

} catch (Exception e) {

Toast.makeText(getApplicationContext(), "Error",

Toast.LENGTH_SHORT).show();

}

break;

}

case R.id.rightEarButton: {

try {

if (rightEarButton.isChecked())

frequencygen.playTone(FrequencyGenerator.RIGHT_EAR);

} catch (Exception e) {

Toast.makeText(getApplicationContext(), "Error",

Toast.LENGTH_SHORT).show();

}

break;

}

case R.id.canHearButton: {

try {

yVals[indexYval] -= 1;

updatePlot();

audioManager.setStreamVolume(AudioManager.STREAM_MUSIC,

AudioManager.ADJUST_LOWER, AudioManager.FLAG_VIBRATE);

} catch (Exception e) {

Toast.makeText(getApplicationContext(), "Error",

Toast.LENGTH_SHORT).show();

}

break;

}

case R.id.cannotHearButton: {

try {

yVals[indexYval] += 1;

updatePlot();

audioManager.adjustStreamVolume(AudioManager.STREAM_MUSIC,

AudioManager.ADJUST_RAISE, AudioManager.FLAG_VIBRATE);

} catch (Exception e) {

Toast.makeText(getApplicationContext(), "Error",

Toast.LENGTH_SHORT).show();

}

break;

}

} // end of switch case

} // end of onClick methodrightEarButton

private void setFreqIndex() {

xVals[0] = 250;

xVals[1] = 500;

xVals[2] = 1000;

xVals[3] = 2000;

xVals[4] = 3000;

xVals[5] = 4000;

xVals[6] = 6000;

xVals[7] = 8000;

//  if(xVals[indexXvals] == xVals[7] ) {

//      xVals[0] = 40;

//  }

}

private void setDecibels() {

//yVals[ indexYval + 1 ] = 40;

//  for (int i = 1; i < DB_LEN; i += 1) {

//yVals[i] = ((int) + yVals[i]);

//  }

// int i = 0;

// yVals[i++] = 40;

updatePlot();

}

private void setFreqValue() {

frequencygen.freqOfTone = frequencies[currentFreq];

if (currentFreq > 6) {

frequencies[0] = currentFreq;

}

currentFreq += 1;

}

// nested class of SimpleXYSeries

public SimpleXYSeries getSeries() {

if (leftEarButton.isChecked()) {

return new SimpleXYSeries(Arrays.asList(xVals),

Arrays.asList(yVals), "Left Ear");

} else {

return new SimpleXYSeries(Arrays.asList(xVals),

Arrays.asList(yVals), "Right Ear");

// same as above:

}

}

/**

*

*/

private void updatePlot() {

// Remove all current series from each plot

Iterator<XYSeries> iterator1 = mySimpleXYPlot.getSeriesSet().iterator();

while (iterator1.hasNext()) {

XYSeries setElement = iterator1.next();

mySimpleXYPlot.removeSeries(setElement);

}

drawBackground();

if (leftEarButton.isChecked()) {

// Create a formatter to use for drawing a series using

// LineAndPointRenderer

// and configure it from xml:

LineAndPointFormatter series1Format = new LineAndPointFormatter();

series1Format.setPointLabelFormatter(new PointLabelFormatter());

series1Format.configure(getApplicationContext(),

R.xml.line_point_formatter_with_plf1);

mySimpleXYPlot.clear();

series1 = (SimpleXYSeries) getSeries(); // call getSeries function

mySimpleXYPlot.addSeries(series1, series1Format);

mySimpleXYPlot.redraw(); // redraw series

}

else {

LineAndPointFormatter series2Format = new LineAndPointFormatter();

series2Format.setPointLabelFormatter(new PointLabelFormatter());

series2Format.configure(getApplicationContext(),

R.xml.line_point_formatter_with_plf2);

mySimpleXYPlot.clear();

series2 = (SimpleXYSeries) getSeries(); // call getSeries function

// add a new series' to the xyplot:

mySimpleXYPlot.addSeries(series2, series2Format);

mySimpleXYPlot.redraw();

}

// Setup our Series with the selected number of elements

// series1 = new SimpleXYSeries(Arrays.asList(series1Numbers),

// SimpleXYSeries.ArrayFormat.Y_VALS_ONLY, "Us");

// series2 = new SimpleXYSeries(Arrays.asList(series2Numbers),

// SimpleXYSeries.ArrayFormat.Y_VALS_ONLY, "Them");

}

private void clearPlot() {

// Remove all current series from each plot

Iterator<XYSeries> iterator1 = mySimpleXYPlot.getSeriesSet().iterator();

while (iterator1.hasNext()) {

XYSeries setElement = iterator1.next();

mySimpleXYPlot.removeSeries(setElement);

}

}

private void drawBackground() {

RectF rect = mySimpleXYPlot.getGraphWidget().getGridRect();

BitmapShader myShader = new BitmapShader(Bitmap.createScaledBitmap(

BitmapFactory.decodeResource(getResources(),

R.drawable.audiogram_background), 1, (int) rect

.height(), false), Shader.TileMode.CLAMP,

Shader.TileMode.CLAMP);

Matrix m = new Matrix();

m.setTranslate(rect.left, rect.top);

myShader.setLocalMatrix(m);

mySimpleXYPlot.getGraphWidget().getGridBackgroundPaint()

.setShader(myShader);

}

public void graphSettings() {

mySimpleXYPlot.setTitle("Audiogram");

mySimpleXYPlot.setRangeLabel("[dB] Hearing level");

mySimpleXYPlot.setDomainLabel("Frequency Hz");

mySimpleXYPlot.setDomainBoundaries(0, 8000, BoundaryMode.FIXED);

mySimpleXYPlot.setRangeBoundaries(120, 0, BoundaryMode.FIXED);

mySimpleXYPlot.setDomainStep(XYStepMode.INCREMENT_BY_VAL, 1000);

// mySimpleXYPlot.setRangeStep(XYStepMode.INCREMENT_BY_VAL, 10);

mySimpleXYPlot.getBackgroundPaint().setColor(Color.WHITE);

mySimpleXYPlot.getGraphWidget().getBackgroundPaint()

.setColor(Color.WHITE);

mySimpleXYPlot.getGraphWidget().getGridBackgroundPaint()

.setColor(Color.WHITE);

mySimpleXYPlot.getGraphWidget().getDomainLabelPaint()

.setColor(Color.BLACK);

mySimpleXYPlot.getGraphWidget().getRangeLabelPaint()

.setColor(Color.BLACK);

mySimpleXYPlot.getGraphWidget().getDomainOriginLabelPaint()

.setColor(Color.BLACK);

mySimpleXYPlot.getGraphWidget().getDomainOriginLinePaint()

.setColor(Color.BLACK);

mySimpleXYPlot.getTitleWidget().getLabelPaint().setColor(Color.BLACK);

mySimpleXYPlot.getTitleWidget().getLabelPaint().setTextSize(15);

mySimpleXYPlot.getTitleWidget().setHeight(30);

mySimpleXYPlot.getTitleWidget().setWidth(400);

mySimpleXYPlot.getGraphWidget().setPaddingTop(8);

mySimpleXYPlot.getGraphWidget().setPaddingBottom(12);

mySimpleXYPlot.getGraphWidget().setPaddingLeft(-8);

mySimpleXYPlot.getGraphWidget().setPaddingRight(18);

mySimpleXYPlot.getDomainLabelWidget().getLabelPaint().setTextSize(15);

mySimpleXYPlot.getDomainLabelWidget().setWidth(100);

mySimpleXYPlot.getDomainLabelWidget().setHeight(18);

mySimpleXYPlot.getDomainLabelWidget().getLabelPaint()

.setColor(Color.BLACK);

mySimpleXYPlot.getRangeLabelWidget().getLabelPaint().setTextSize(15);

mySimpleXYPlot.getRangeLabelWidget().setWidth(50);

mySimpleXYPlot.getRangeLabelWidget().setHeight(500);

mySimpleXYPlot.getRangeLabelWidget().getLabelPaint()

.setColor(Color.BLACK);

}

public void startUp() {

AlertDialog.Builder alertbox = new AlertDialog.Builder(

HearingTestActivity.this);

alertbox.setIcon(R.drawable.ic_launcher);

alertbox.setMessage("You will hear a series of tones for 5 seconds each."

+ "\n\nSelect whether you can hear or cannot hear"

+ "\n\nAt clearest dB you can hear a tone then choose select");

alertbox.setTitle("Getting started");

alertbox.setPositiveButton("Ok", new DialogInterface.OnClickListener() {

public void onClick(DialogInterface arg0, int arg1) {

// finish used for destroyed activity

}

});

alertbox.show();

}

// Method for alert dialog, when user exits

public boolean onKeyDown(int keyCode, KeyEvent event) {

if ((keyCode == KeyEvent.KEYCODE_BACK)) {

AlertDialog.Builder alertbox = new AlertDialog.Builder(

HearingTestActivity.this);

alertbox.setIcon(R.drawable.ic_launcher);

alertbox.setTitle("Cancel the test?");

alertbox.setPositiveButton("Yes",

new DialogInterface.OnClickListener() {

public void onClick(DialogInterface arg0, int arg1) {

// finish used for destroyed activity

finish();

}

});

alertbox.setNegativeButton("No",

new DialogInterface.OnClickListener() {

public void onClick(DialogInterface arg0, int arg1) {

Toast.makeText(getApplicationContext(),

"Test continue", Toast.LENGTH_SHORT).show();

}

});

alertbox.show();

}

return super.onKeyDown(keyCode, event);

}

// onDestroy called when app exited

// we stop app from playing tone in background

@Override

protected void onDestroy() {

super.onDestroy();

Log.d("HearingTest", "onDestroy called");

try {

frequencygen.audioTrack.release();

frequencygen.audioTrack = null;

frequencygen = null;

} catch (Exception e) {

Log.e("HearingTest", "onPause error: " + e.toString());

}

}

// onPause is when user leaves the current activity

// so we stop frequencygen class from generating tone

@Override

protected void onPause() {

super.onPause();

Log.d("HearingTest", "onPause called");

try {

frequencygen.audioTrack.release();

frequencygen.audioTrack = null;

frequencygen = null;

} catch (Exception e) {

Log.e("HearingTest", "onPuse error: " + e.toString());

}

}

} // end of class

### MoreInfo.java

/**

* {This class runs the info activity and launches new intent based on what user chooses}

*

* @version Build {0.6} (26 March 2015)

* @author Junaid Malik

*/

package com.example.testtonegenv10;

import android.app.Activity;

import android.content.Intent;

import android.os.Bundle;

import android.view.View;

public class MoreInfo extends Activity {

@Override

public void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

setContentView(R.layout.moreinfo);

}

public void loadView1(View v) {

Intent intent = new Intent(MoreInfo.this, InfoView1.class);

startActivity(intent);

}

public void loadView2(View v) {

Intent intent = new Intent(MoreInfo.this, InfoView2.class);

startActivity(intent);

}

public void loadView3(View v) {

Intent intent = new Intent(MoreInfo.this, InfoView3.class);

startActivity(intent);

}

public void loadView4(View v) {

Intent intent = new Intent(MoreInfo.this, InfoView4.class);

startActivity(intent);

}

}

### MoreInfo2.java

package com.example.testtonegenv10;

import android.app.Activity;

import android.os.Bundle;

public class MoreInfo2 extends Activity {

@Override

public void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

setContentView(R.layout.moreinfo2);

}

}

### StartTest.java

package com.example.testtonegenv10;

import android.app.Activity;

import android.content.Intent;

import android.os.Bundle;

import android.view.View;

public class StartTest extends Activity {

@Override

public void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

setContentView(R.layout.calibration_info);

}

public void startTest(View v)

{

Intent intent = new Intent(this, HearingTestActivity.class);

startActivity(intent);

}

}

### InfoView1.java

/*

* {Simple class loads infoView1.java}

*

* @version Build {0.6} (26 March 2015)

* @author Junaid Malik

*/

package com.example.testtonegenv10;

import android.app.Activity;

import android.os.Bundle;

import android.view.View;

import android.view.View.OnClickListener;

public class InfoView1 extends Activity implements OnClickListener {

@Override

public void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

setContentView(R.layout.infoview1);

}

@Override

public void onClick(View v) {

// TODO Auto-generated method stub

}

}

### InfoView2.java

/*

* {Simple class loads infoView2.java}

*

* @version Build {0.6} (26 March 2015)

* @author Junaid Malik

*/

package com.example.testtonegenv10;

import android.app.Activity;

import android.os.Bundle;

import android.view.View;

import android.view.View.OnClickListener;

public class InfoView2 extends Activity {

@Override

public void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

setContentView(R.layout.infoview2);

}

}

### InfoView3.java

package com.example.testtonegenv10;

import android.app.Activity;

import android.os.Bundle;

public class InfoView3 extends Activity {

@Override

public void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

setContentView(R.layout.infoview3);

}

}

### InfoView4.java

package com.example.testtonegenv10;

import com.example.testtonegenv10.R;

import android.app.Activity;

import android.os.Bundle;

public class InfoView4 extends Activity {

@Override

public void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

setContentView(R.layout.infoview4);

}

}

## Appendix F: Log file of changes

Junaid Malik - Final Year Project 2015 Dublin Institute of Tech. - Hearing Test App

------------------- MAIN PROJECT IS SAVED IN GIT HUB, BACKUP IN WORKSPACE ---------------

Changelog Legend:

-----------------------------------------------------------------------------------------

[+] = Added

[*] = Changed

[^] = Moved

[=] = No Changes

[x] = Deleted

[!] = Bugs

------------------------------------------------------------------------------------------

(04/05/15) - Improvements - Version 1.09

[+] - increment for frequency array values

[*] - change next frequency button to have method setFreqValue

------------------------------------------------------------------------------------------

(29/04/15) - Improvements - Version 1.08

[+] - increments yVals with simple (+= 1) etc

[+] - xVals are fixed and do need to be changed

[+] - added "rate this app" plug in by Keisuke Kobayashi - added library and main code

[+] - added strings for "rate this app"

[+] - added indexYval to control index position

[+] - next frequency always set 40

[+] - added strings and xml layout for morinfoview 2, also added to main menu

[+] - added info and new icon to main menu

------------------------------------------------------------------------------------------

(21/04/15) - Improvements - Version 1.05

[+] - added x and y graph to be updated using SimpleXYSeries

[*] - formatted java files and xml files with cleaner layout

[*] - updated app icons and image buttons

[*] - added onPause and onDestroy methods for main class to stop playing tone in background

[*] - changed headphone_info to calibration_info

[+] - added a custom SimpleXYSeries, removed code that limits x,y size

[+] - added preset frequency values as they never change

[+] - added colors.xml(values folder), nice_button.xml and a drawable folder

[+] - removed multiple launchers from android manifest

------------------------------------------------------------------------------------------

(17/04/15) - Improvements - Version 1.0

[+] - Added radio buttons controlling left and right ear.

[+] - Updated icons with much cleaner look using MS Paint.

[+] - Added project to git hub.

[+] - Added list view of audiograms to load button

user can view old tests he has done and view them in a list

[+] - Added infoview4.xml which contains contact information.

[+] - Added SimpleXYSeries getSeries() method to add series which can be changed by user.

------------------------------------------------------------------------------------------

(31/03/15) - Improvements - Version 0.8 - 0.9

[+] - Fixed fragment issue by correct id and class

[+] - FragmentAudiogram does nothing, and FragmentAudiogram only for inflating view

[*] - HeaingTestActivity contains implementation for both Audiogram and PlayTone fragments.

[+] - Added StartTest.java and headphone_info.xml before main HearingTestActivity.java

[*] - Fix and re-arranged Strings file to properly allocate id's

[+] -

------------------------------------------------------------------------------------------

(30/03/15) - Improvements - Version 0.7

[!] - App was crashing when fragment tag was added.

[!] - When removed it worked, so failure is when app renders fragment element

[+] - Fixed issue by importing correct android support library

[*] - Files rename to FragmentPlayTone.java and playtone_fragment.xml

------------------------------------------------------------------------------------------

(25/03/15) - Improvements - Version 0.6

[*] - Audiogram.java and FrequencyGen.java have seperate xml file,

audiogram_fragment and frequency_fragment

[+] - Added hearingtest_view which contains audiogram and frequency fragments

[*] - Refactored class name of Audiogram, HearingTest and created MainFragment

class FragmentHearingTest

------------------------------------------------------------------------------------------

(23/03/15) - Improvements - Version 0.5

[+] - Created new classes for Audiogram, FrequencyGen and HearingTest.

[*] - Frequency Tone now generated in its own class FrequencyGen.java

[*] - Audiogram graph is now generated by Audiogram.java

[*] - Test logic is to be designed in HearingTest.java

[*] - each class should have own function.

[!] - Unable to have both classes acting on one layout

------------------------------------------------------------------------------------------

(23/02/15) - Improvements - Version 0.4

[+] - Implemented AndroidPlot library for XYPlot graph.

[+] - Improve comply to coding standards.

[!] - Unable to get graph to change as user presses button.

[!] -

------------------------------------------------------------------------------------------

(9/2/15) - Improvements - Version 0.3

[+] - Added MainMenu class.

[+] - Added activities for Information on hearing.

[+] - SplashScreen at start.

------------------------------------------------------------------------------------------

(1/2/15) - Improvements - Version 0.2

[+] - Class to generate simple pure tones added.

[+] - added buttons to play different tones.

------------------------------------------------------------------------------------------

(1/2/15) - Improvements - Version 0.1

------------------------------------------------------------------------------------------

[+] - Project created.

[+] - Sample projects tested.

------------------------------------------------------------------------------------------

## Appendix G: The AndroidManifest.xml

<?xml version="1.0" encoding="utf-8"?>

<manifest xmlns:android="http://schemas.android.com/apk/res/android"

package="com.example.testtonegenv10"

android:versionCode="10"

android:versionName="10" >

<uses-sdk

android:minSdkVersion="11"

android:targetSdkVersion="16" />

<application

android:allowBackup="true"

android:icon="@drawable/ic_launcher"

android:label="@string/app_name"

android:theme="@style/AppTheme" >

<activity

android:name="com.example.testtonegenv10.MainMenu"

android:label="@string/app_name" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.LAUNCHER" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.StartTest"

android:label="@string/start_test" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.HearingTestActivity"

android:label="@string/testing_screen" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.FragmentFrequency"

android:label="@string/testing_screen" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.FrequencyGenerator"

android:label="@string/testing_screen" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.FragmentAudiogram"

android:label="@string/testing_screen" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.LoadTestActivity"

android:label="@string/loadprev_test" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.MoreInfo"

android:label="@string/moreinfo_activity" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.MoreInfo2"

android:label="@string/moreinfo2" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.InfoView1"

android:label="@string/Infoview1" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.InfoView2"

android:label="@string/Infoview2" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.InfoView3"

android:label="@string/Infoview3" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

<activity

android:name="com.example.testtonegenv10.InfoView4"

android:label="@string/Infoview4" >

<intent-filter>

<action android:name="android.intent.action.MAIN" />

<category android:name="android.intent.category.EMBED" />

</intent-filter>

</activity>

</application>

</manifest>

## Appendix H: Strings Resource file

<?xml version="1.0" encoding="utf-8"?>

<resources>

<string name="app_name">TestToneGen V10</string>

<string name="splash">SplashScreen</string>

<string name="begin">Begin a test</string>

<string name="load">Load test</string>

<string name="about">About</string>

<string name="frequency_fragment">Frequency fragment</string>

<string name="audiogram_fragment">Audiogram fragment</string>

<string name="moreinfo_activity">Information on hearing</string>

<string name="moreinfo2">Human Auditory System</string>

<string name="Infoview1">What is hearing loss?</string>

<string name="Infoview2">Symptoms of hearing loss</string>

<string name="Infoview3">What is a hearing test?</string>

<string name="Infoview4">About developer</string>

<string name="audiogram_screen">This screen displays the results</string>

<string name="hello_world">\t\tWelcome to the \n\t\tHearing Test App</string>

<string name="testing_screen">Test Screen</string>

<string name="start_test">Test starting.</string>

<string name="calibrate_title">Calibration</string>

<string name="calibrate_info">For best results, its highly recommended that you use    headphones for the remainder of this test.</string>

<string name="calibrate_button">Click here to continue</string>

<string name="loadprev_test">Load a test</string>

<string name="loadtest_title_hint">Enter a title for the test</string>

<string name="load_title_label">Title</string>

<string name="load_test_date">Date:</string>

<string name="load_test_checked">Checked:</string>

<string name="load_test_title_hint">Enter a name for the test.</string>

<string name="action_settings">Settings</string>

<string name="button1">500hz</string>

<string name="button2">1kHz</string>

<string name="button3">2kHz</string>

<string name="button4">3kHz</string>

<string name="button5">4kHz</string>

<string name="button6">Begin a test</string>

<string name="leftEar">Left</string>

<string name="rightEar">Right</string>

<string name="infobutton1">What is hearing loss?</string>

<string name="infobutton2">Symptoms of hearing loss</string>

<string name="infobutton3">What is the test?</string>

<string name="infobutton4">About developer</string>

<string name="okbutton">OK</string>

<string name="infoView1">\n•Hearing is the ability to perceive sounds. Sound occurs over a wide spectrum of frequencies.

The human ear is sensitive to a frequency band within that spectrum expressed in decibels

(dB).

\n\n•Frequencies capable of being heard by humans are called audio or sonic. The range is

typically considered to be between 20 Hz and 20 ,000 Hz (Hertz).

\n\n•Frequencies higher than

audio are referred to as ultrasonic, while frequencies below audio are referred to as

infrasonic. Loss of the ability to hear sound frequencies in the normal range of hearing is

called hearing impairment.\n\n</string>

<string name="infoView2">\nYou may have a hearing loss if:

\n\n•You hear people speaking but you have difficulty understanding what they are saying

\n\n•You are frequently having to ask people to repeat what they have said

\n\n•You feel left out of conversations

\n\n•You complain that others are mumbling

\n\n•You need to ask colleagues about the details of a meeting or presentation you attended

\n\n•You miss jokes because you didn\'t hear all of what was said

\n\n•Friends and family need you to turn the TV or radiodown

\n\n•You do not hear the telephone and or doorbell

\n\n•You have difficulty working out where sounds are coming from

\n\n•You have a history of exposure to loud noise e.g. working in the construction industry

\n\n•You understand more of what is being said if you are looking at the person speaking to you

\n\n•Soft sounds seem to have disappeared altogether in speech</string>

<string name="infoView3">\n•Pure tone audiometry is the standard assesment of an individual\'s hearing.\n\n•The results of a pure tone audiometry test are recorded onto a chart called an audiogram.

\n\n•It is an easy method to obtain and provide information about an individuals hearing across a range of frequencies.

\n\n•A pure tone is a tone that has a single specific frequency.This tone is determined by the rate, or speed at which the sound waves vibrate.

\n\n•A 4000 Hz tone has a higher frequency than the 500 Hz tone.</string>

<string name="infoView4">The system of the body that lets a person hear sound is called the auditory system. It is made up of four main parts.

The visible part of the ear and the ear canal make up the outer ear.

The area behind the eardrum is called the middle ear. It is made up of the three smallest bones in the body (malleus, incus and stapes) and the Eustachian tube, which leads from the middle ear to the back of the throat.

\n\nThe main organ for hearing, the  cochlea, is in the inner ear. The cochlea is the sense organ for hearing like the nose is for smelling. The middle ear bones are connected to the cochlea and the cochlea is connected to the  auditory nerve.

The auditory nerve is the nerve that runs from the inner ear (cochlea) to the brainstem. The cochlea sends signals to the auditory nerve; then the auditory nerve sends these signals to the brain.

Hearing loss can happen because of problems in any part of the auditory system.</string>

<string name="infoView5">Hearing is different than other senses like vision, smell and taste. The hearing system is about changing movement into a signal that the brain can understand. Sound travels through the air as vibrations in the air pressure (i.e. sound waves). It takes four steps through the auditory system before the brain can understand sounds.

\n\nStep 1: Sound moves into the ear through the outer ear and the ear canal. Once the sound has moved down the ear canal, the sound waves hit the eardrum, which makes the eardrum vibrate.

\n\nStep 2: When the eardrum vibrates, it moves the bones of the middle part of the ear (malleus, incus and stapes). The movement of the malleus and incus makes the stapes move in and out of the entrance of the inner ear, the oval window.

\n\nStep 3: The other side of the oval window, in the inner ear, is filled with fluid. As the stapes moves back and forth on the oval window, it makes waves. This changes the vibrations of the middle ear into hydraulic energy.

\n\nStep 4: The hydraulic energy of the waves makes tiny hair cells in the  cochlea sway back and forth. The swaying action creates signals, which move through the  auditory nerve then up the levels of the brainstem until they reach the brain.</string>

<string name="rta_dialog_title">Rate this app</string>

<string name="rta_dialog_message">If you enjoy this app, please take a moment to rate this app. It won\'t take more than a minute. Thank you for your support!</string>

<string name="rta_dialog_ok">Rate now</string>

<string name="rta_dialog_cancel">Remind later</string>

<string name="rta_dialog_no">No, thanks</string>

</resources>

## Appendix I: Layout Files

### calibration_info.xml

<?xml version="1.0" encoding="utf-8"?>

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"

android:layout_width="match_parent"

android:layout_height="match_parent"

android:background="#FFFFFF"

android:gravity="center" >

<TextView

android:id="@+id/textView2"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentTop="true"

android:layout_centerHorizontal="true"

android:layout_marginTop="23dp"

android:gravity="center"

android:text="@string/calibrate_title"

android:textAppearance="?android:attr/textAppearanceLarge"

android:textSize="50sp" />

<TextView

android:id="@+id/textView1"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentLeft="true"

android:layout_alignParentRight="true"

android:layout_below="@+id/textView2"

android:layout_marginTop="32dp"

android:gravity="center"

android:text="@string/calibrate_info"

android:textAppearance="?android:attr/textAppearanceLarge" />

<Button

android:id="@+id/button1"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignLeft="@+id/textView2"

android:layout_alignParentBottom="true"

android:layout_alignRight="@+id/textView2"

android:layout_marginBottom="32dp"

android:background="@drawable/nice_button"

android:gravity="center"

android:onClick="startTest"

android:shadowColor="@android:color/black"

android:text="@string/calibrate_button"

android:textColor="@android:color/white" />

<ImageView

android:id="@+id/imageView1"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignRight="@+id/button1"

android:layout_below="@+id/textView1"

android:background="#FFFFFF"

android:src="@drawable/head_phones_img" />

</RelativeLayout>

### infoview1.xml

<?xml version="1.0" encoding="utf-8"?>

<ScrollView xmlns:android="http://schemas.android.com/apk/res/android"

android:layout_width="wrap_content"

android:layout_height="wrap_content" >

<RelativeLayout

android:layout_width="match_parent"

android:layout_height="wrap_content"

>

<TextView

android:id="@+id/infoView1"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentLeft="true"

android:layout_alignParentTop="true"

android:gravity="center_vertical"

android:text="@string/infoView1"

android:textSize="20sp" />

</RelativeLayout>

</ScrollView>

### infoview2.xml

<?xml version="1.0" encoding="utf-8"?>

<ScrollView xmlns:android="http://schemas.android.com/apk/res/android"

android:layout_width="wrap_content"

android:layout_height="wrap_content" >

<RelativeLayout

android:layout_width="match_parent"

android:layout_height="wrap_content"

>

<TextView

android:id="@+id/infoView2"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentTop="true"

android:text="@string/infoView2"

android:textSize="20sp" />

</RelativeLayout>

</ScrollView>

### infoview3.xml

<?xml version="1.0" encoding="utf-8"?>

<ScrollView xmlns:android="http://schemas.android.com/apk/res/android"

xmlns:tools="http://schemas.android.com/tools"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

tools:context=".InfoView3" >

<RelativeLayout

android:layout_width="match_parent"

android:layout_height="wrap_content" >

<TextView

android:id="@+id/infoView3"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentTop="true"

android:text="@string/infoView3"

android:textSize="20sp" />

</RelativeLayout>

</ScrollView>

### infoview4.xml

<?xml version="1.0" encoding="utf-8"?>

<RelativeLayout xmlns:android="http://schemas.android.com/apk/res/android"

android:layout_width="match_parent"

android:layout_height="match_parent"

android:orientation="vertical" >

<ImageView

android:id="@+id/imageView1"

android:layout_width="wrap_content"

android:layout_height="match_parent"

android:scaleType="centerCrop"

android:src="@drawable/infoview4" />

<TextView

android:id="@+id/TextView03"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_alignParentBottom="true"

android:layout_centerHorizontal="true"

android:layout_marginBottom="37dp"

android:text="junmalik109@gmail.com"

android:textColor="#FFFFFF"

android:textSize="20dp" />

<TextView

android:id="@+id/textview"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_above="@+id/TextView02"

android:layout_centerHorizontal="true"

android:text="Hearing Test App"

android:textColor="#FFFFFF"

android:textSize="25dp" />

<TextView

android:id="@+id/TextView01"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_above="@+id/textview"

android:layout_centerHorizontal="true"

android:text="Version 1.0"

android:textColor="#FFFFFF"

android:textSize="25dp" />

<TextView

android:id="@+id/TextView02"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:layout_above="@+id/TextView03"

android:layout_centerHorizontal="true"

android:layout_marginBottom="49dp"

android:text="© 2015 - Junaid Malik"

android:textColor="#FFFFFF"

android:textSize="25dp" />

</RelativeLayout>

### loadtest.xml

<?xml version="1.0" encoding="utf-8"?>

<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"

android:layout_width="fill_parent"

android:layout_height="fill_parent" >

<com.androidplot.xy.XYPlot

android:id="@+id/xyplot"

android:layout_width="fill_parent"

android:layout_height="250dp"

title="an xy plot" />

</LinearLayout>

### loadprev_test.xml

<?xml version="1.0" encoding="utf-8"?>

<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"

android:id="@+id/LinearLayout01"

android:layout_width="fill_parent"

android:layout_height="fill_parent" >

<ListView

android:id="@+id/listaudiogram"

android:layout_width="fill_parent"

android:layout_height="fill_parent" />

</LinearLayout>

## Appendix J: Open Source Libraries

### SimpleXYSeries.java by AndroidPlot developers

/*

* Copyright 2012 AndroidPlot.com

*

*    Licensed under the Apache License, Version 2.0 (the "License");

*    you may not use this file except in compliance with the License.

*    You may obtain a copy of the License at

*

*        http://www.apache.org/licenses/LICENSE-2.0

*

*    Unless required by applicable law or agreed to in writing, software

*    distributed under the License is distributed on an "AS IS" BASIS,

*    WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

*    See the License for the specific language governing permissions and

*    limitations under the License.

*/

// this class has been modified by Junaid Malik

// checking of value length to be the same size has been removed

package com.example.testtonegenv10;

import android.graphics.Canvas;

import android.util.Log;

import android.util.Pair;

import com.androidplot.Plot;

import com.androidplot.PlotListener;

import com.androidplot.xy.XYSeries;

import java.util.LinkedList;

import java.util.List;

import java.util.NoSuchElementException;

import java.util.concurrent.locks.ReentrantReadWriteLock;

/**

* A convenience class used to create instances of XYPlot generated from Lists of Numbers.

*/

public class SimpleXYSeries implements XYSeries, PlotListener {

private static final String TAG = SimpleXYSeries.class.getName();

@Override

public void onBeforeDraw(Plot source, Canvas canvas) {

lock.readLock().lock();

}

@Override

public void onAfterDraw(Plot source, Canvas canvas) {

lock.readLock().unlock();

}

public enum ArrayFormat {

Y_VALS_ONLY,

XY_VALS_INTERLEAVED

}

private volatile LinkedList<Number> xVals = new LinkedList<Number>();

private volatile LinkedList<Number> yVals = new LinkedList<Number>();

private volatile String title = null;

private ReentrantReadWriteLock lock = new ReentrantReadWriteLock(true);

public SimpleXYSeries(String title) {

this.title = title;

}

/**

* Generates an XYSeries instance from the List of numbers passed in.  This is a convenience class

* and should only be used for static data models; it is not suitable for representing dynamically

* changing data.

*

* @param model  A List of Number elements comprising the data model.

* @param format Format of the model.  A format of Y_VALS_ONLY means that the array only contains y-values.

*               For this format x values are autogenerated using values of 0 through n-1 where n is the size of the model.

* @param title  Title of the series

*/

public SimpleXYSeries(List<? extends Number> model, ArrayFormat format, String title) {

this(title);

setModel(model, format);

}

public SimpleXYSeries(List<? extends Number> xVals, List<? extends Number> yVals, String title) {

this(title);

if(xVals == null || yVals == null) {

throw new IllegalArgumentException("Neither the xVals nor the yVals parameters may be null.");

}

// if(xVals.size() != yVals.size()) {

//      throw new IllegalArgumentException("xVals and yVals List parameters must be of the same size.");

// }

this.xVals.addAll(xVals);

this.yVals.addAll(yVals);

}

/**

* Use index value as xVal, instead of explicit, user provided xVals.

*/

public void useImplicitXVals() {

lock.writeLock().lock();

try {

xVals = null;

} finally {

lock.writeLock().unlock();

}

}

/**

* Use the provided list of Numbers as yVals and their corresponding indexes as xVals.

* @param model A List of Number elements comprising the data model.

* @param format Format of the model.  A format of Y_VALS_ONLY means that the array only contains y-values.

*               For this format x values are autogenerated using values of 0 through n-1 where n is the size of the model.

*/

public void setModel(List<? extends Number> model, ArrayFormat format) {

lock.writeLock().lock();

try {

// empty the current values:

//xVals.clear();

xVals = null;

yVals.clear();

// make sure the new model has data:

if (model == null || model.size() == 0) {

return;

}

switch (format) {

// array containing only y-vals. assume x = index:

case Y_VALS_ONLY:

for(Number n : model) {

yVals.add(n);

}

/*for (int i = 0; i < model.size(); i++) {

//xVals.add(i);

yVals.add(model.get(i));

}*/

break;

// xy interleaved array:

case XY_VALS_INTERLEAVED:

if (xVals == null) {

xVals = new LinkedList<Number>();

}

if (model.size() % 2 != 0) {

throw new IndexOutOfBoundsException("Cannot auto-generate series from odd-sized xy List.");

}

// always need an x and y array so init them now:

int sz = model.size() / 2;

for (int i = 0, j = 0; i < sz; i++, j += 2) {

xVals.add(model.get(j));

yVals.add(model.get(j + 1));

}

break;

default:

throw new IllegalArgumentException("Unexpected enum value: " + format);

}

} finally {

lock.writeLock().unlock();

}

}

/**

* Sets individual x value based on index

* @param value

* @param index

*/

public void setX(Number value, int index) {

lock.writeLock().lock();

try {

xVals.set(index, value);

} finally {

lock.writeLock().unlock();

}

}

/**

* Sets individual y value based on index

* @param value

* @param index

*/

public void setY(Number value, int index) {

lock.writeLock().lock();

try {

yVals.set(index, value);

} finally {

lock.writeLock().unlock();

}

}

/**

* Sets xy values based on index

* @param xVal

* @param yVal

* @param index

*/

public void setXY(Number xVal, Number yVal, int index) {

lock.writeLock().lock();

try {

yVals.set(index, yVal);

xVals.set(index, xVal);

} finally {lock.writeLock().unlock();}

}

public void addFirst(Number x, Number y) {

lock.writeLock().lock();

try {

if (xVals != null) {

xVals.addFirst(x);

}

yVals.addFirst(y);

} finally {

lock.writeLock().unlock();

}

}

/**

*

* @return Pair<Number, Number> with first equal to x-val and second equal to y-val.

*/

public Pair<Number, Number> removeFirst() {

lock.writeLock().lock();

try {

if (size() <= 0) {

throw new NoSuchElementException();

}

return new Pair<Number, Number>(xVals != null ? xVals.removeFirst() : 0, yVals.removeFirst());

} finally {

lock.writeLock().unlock();

}

}

public void addLast(Number x, Number y) {

lock.writeLock().lock();

try {

if (xVals != null) {

xVals.addLast(x);

}

yVals.addLast(y);

} finally {

lock.writeLock().unlock();

}

}

/**

*

* @return Pair<Number, Number> with first equal to x-val and second equal to y-val.

*/

public Pair<Number, Number> removeLast() {

lock.writeLock().lock();

try {

if (size() <= 0) {

throw new NoSuchElementException();

}

return new Pair<Number, Number>(xVals != null ? xVals.removeLast() : yVals.size() - 1, yVals.removeLast());

} finally {

lock.writeLock().unlock();

}

}

@Override

public String getTitle() {

return title;

}

public void setTitle(String title) {

lock.writeLock().lock();

try {

this.title = title;

} finally {lock.writeLock().unlock();}

}

@Override

public int size() {

return yVals != null ? yVals.size() : 0;

}

@Override

public Number getX(int index) {

return xVals != null ? xVals.get(index) : index;

}

@Override

public Number getY(int index) {

return yVals.get(index);

}

}

### RateThisApp.java by Keisuke Kobayashi

/*

* Copyright 2013 Keisuke Kobayashi

*

* Licensed under the Apache License, Version 2.0 (the "License");

* you may not use this file except in compliance with the License.

* You may obtain a copy of the License at

*

*     http://www.apache.org/licenses/LICENSE-2.0

*

* Unless required by applicable law or agreed to in writing, software

* distributed under the License is distributed on an "AS IS" BASIS,

* WITHOUT WARRANTIES OR CONDITIONS OF ANY KIND, either express or implied.

* See the License for the specific language governing permissions and

* limitations under the License.

*/

package com.kskkbys.rate;

import java.util.Date;

import com.example.testtonegenv10.R;

import android.app.AlertDialog;

import android.content.Context;

import android.content.DialogInterface;

import android.content.DialogInterface.OnCancelListener;

import android.content.DialogInterface.OnClickListener;

import android.content.Intent;

import android.content.SharedPreferences;

import android.content.SharedPreferences.Editor;

import android.net.Uri;

import android.util.Log;

/**

* RateThisApp<br>

* A library to show the app rate dialog

* @author Keisuke Kobayashi <k.kobayashi.122@gmail.com>

*

*/

public class RateThisApp {

private static final String TAG = RateThisApp.class.getSimpleName();

private static final String PREF_NAME = "RateThisApp";

private static final String KEY_INSTALL_DATE = "rta_install_date";

private static final String KEY_LAUNCH_TIMES = "rta_launch_times";

private static final String KEY_OPT_OUT = "rta_opt_out";

private static Date mInstallDate = new Date();

private static int mLaunchTimes = 0;

private static boolean mOptOut = false;

/**

* Days after installation until showing rate dialog

*/

public static final int INSTALL_DAYS = 7;

/**

* App launching times until showing rate dialog

*/

public static final int LAUNCH_TIMES = 10;

/**

* If true, print LogCat

*/

public static final boolean DEBUG = false;

/**

* Call this API when the launcher activity is launched.<br>

* It is better to call this API in onStart() of the launcher activity.

*/

public static void onStart(Context context) {

SharedPreferences pref = context.getSharedPreferences(PREF_NAME, Context.MODE_PRIVATE);

Editor editor = pref.edit();

// If it is the first launch, save the date in shared preference.

if (pref.getLong(KEY_INSTALL_DATE, 0) == 0L) {

Date now = new Date();

editor.putLong(KEY_INSTALL_DATE, now.getTime());

log("First install: " + now.toString());

}

// Increment launch times

int launchTimes = pref.getInt(KEY_LAUNCH_TIMES, 0);

launchTimes++;

editor.putInt(KEY_LAUNCH_TIMES, launchTimes);

log("Launch times; " + launchTimes);

editor.commit();

mInstallDate = new Date(pref.getLong(KEY_INSTALL_DATE, 0));

mLaunchTimes = pref.getInt(KEY_LAUNCH_TIMES, 0);

mOptOut = pref.getBoolean(KEY_OPT_OUT, false);

printStatus(context);

}

/**

* Show the rate dialog if the criteria is satisfied

* @param context

*/

public static void showRateDialogIfNeeded(final Context context) {

if (shouldShowRateDialog()) {

showRateDialog(context);

}

}

/**

* Check whether the rate dialog shoule be shown or not

* @return

*/

private static boolean shouldShowRateDialog() {

if (mOptOut) {

return false;

} else {

if (mLaunchTimes >= LAUNCH_TIMES) {

return true;

}

long threshold = INSTALL_DAYS * 24 * 60 * 60 * 1000L;   // msec

if (new Date().getTime() - mInstallDate.getTime() >= threshold) {

return true;

}

return false;

}

}

/**

* Show the rate dialog

* @param context

*/

public static void showRateDialog(final Context context) {

AlertDialog.Builder builder = new AlertDialog.Builder(context);

builder.setTitle(R.string.rta_dialog_title);

builder.setMessage(R.string.rta_dialog_message);

builder.setPositiveButton(R.string.rta_dialog_ok, new OnClickListener() {

@Override

public void onClick(DialogInterface dialog, int which) {

String appPackage = context.getPackageName();

Intent intent = new Intent(Intent.ACTION_VIEW, Uri.parse("https://play.google.com/store/apps/details?id=" + appPackage));

context.startActivity(intent);

setOptOut(context, true);

}

});

builder.setNeutralButton(R.string.rta_dialog_cancel, new OnClickListener() {

@Override

public void onClick(DialogInterface dialog, int which) {

clearSharedPreferences(context);

}

});

builder.setNegativeButton(R.string.rta_dialog_no, new OnClickListener() {

@Override

public void onClick(DialogInterface dialog, int which) {

setOptOut(context, true);

}

});

builder.setOnCancelListener(new OnCancelListener() {

@Override

public void onCancel(DialogInterface dialog) {

clearSharedPreferences(context);

}

});

builder.create().show();

}

/**

* Clear data in shared preferences.<br>

* This API is called when the rate dialog is approved or canceled.

* @param context

*/

private static void clearSharedPreferences(Context context) {

SharedPreferences pref = context.getSharedPreferences(PREF_NAME, Context.MODE_PRIVATE);

Editor editor = pref.edit();

editor.remove(KEY_INSTALL_DATE);

editor.remove(KEY_LAUNCH_TIMES);

editor.commit();

}

/**

* Set opt out flag. If it is true, the rate dialog will never shown unless app data is cleared.

* @param context

* @param optOut

*/

private static void setOptOut(final Context context, boolean optOut) {

SharedPreferences pref = context.getSharedPreferences(PREF_NAME, Context.MODE_PRIVATE);

Editor editor = pref.edit();

editor.putBoolean(KEY_OPT_OUT, optOut);

editor.commit();

}

/**

* Print values in SharedPreferences (used for debug)

* @param context

*/

private static void printStatus(final Context context) {

SharedPreferences pref = context.getSharedPreferences(PREF_NAME, Context.MODE_PRIVATE);

log("*** RateThisApp Status ***");

log("Install Date: " + new Date(pref.getLong(KEY_INSTALL_DATE, 0)));

log("Launch Times: " + pref.getInt(KEY_LAUNCH_TIMES, 0));

log("Opt out: " + pref.getBoolean(KEY_OPT_OUT, false));

}

/**

* Print log if enabled

* @param message

*/

private static void log(String message) {

if (DEBUG) {

Log.v(TAG, message);

}

}

}

# Bibliography

[1] Pure tone air and bone conduction threshold audiometry with and without masking - British Society of Audiology. 2015. Available at: http://www.thebsa.org.uk/resources/pure-tone-air-bone-conduction-threshold-audiometry-without-masking/

[Accessed 20 May 2015]

[2] Suzanne Purdy Head of Speech Science University of Auckland, Guideline for diagnosing occupational noise induced hearing loss Part 3: Audiometric standards Available at: http://www.acc.co.nz/PRD_EXT_CSMP/groups/external_communications/documents/reference_tools/wpc091007.pdf

[Accessed 20 May 2015]

[3] Hearing Tests (Audiometry) | Health | Patient.co.uk. 2015. Hearing Tests (Audiometry) | Health | Patient.co.uk. Available at: http://www.patient.co.uk/health/hearing-tests-audiometry

[Accessed 20 May 2015]

[4] Types of Tests Used to Evaluate Hearing in Children and Adults. Available at: http://www.asha.org/public/hearing/Types-of-Tests-Used-to-Evaluate-Hearing/

[Accessed 20 May 2015]

[5] Androidplot — Bitbucket. Available at: https://bitbucket.org/androidplot/androidplot

[Accessed 20 May 2015]

[6] Marble Mice: Generate And Play A Tone In Android. Available at: http://www.marblemice.blogspot.ie/2010/04/generate-and-play-tone-in-android.html

[Accessed 26 May 2015]

[7] Dr Béatrice Duthey, Ph.D., 20th February2013, Priority Medicines for Europe and the World, “A Public Health Approach to Innovation", Update on 2004 Background Paper, 6.21 Hearing Loss

Available at: http://www.who.int/medicines/areas/priority_medicines/BP6_21Hearing.pdf

[Accessed 26 May 2015]

[8] Application Development for Android Available at: http://www.who.int/medicines/areas/priority_medicines/BP6_21Hearing.pdf

[Accessed 28 May 2015]

[9] Prevalence of age-related hearing loss in Europe: a review – Springer. Available at: http://link.springer.com/article/10.1007%2Fs00405-011-1597-8.

[Accessed 28 May 2015]

[10] Evaluation of the Social and Economic Costs of Hearing Impairment by Shield (2006 - Google Scholar. 2015. Available at: https://scholar.google.com/scholar?q=Evaluation+of+the+Social+and+Economic+Costs+of+Hearing+Impairment%E2%80%9D+by+Shield+

[Accessed 28 May 2015]

[11] Human Ear Diagram Available at: http://www.rudyard.org/wpcontent/uploads/2013/09/human-ear-diagram.jpg

[Accessed 28 May 2015]

[12] Pete Goodliffe, 2014. Becoming a Better Programmer: A Handbook for People Who Care About Code. 1 Edition. O'Reilly Media.

[13] Marko Gargenta, 2011. Learning Android. 1 Edition. O'Reilly Media.

[14] Portable Digital Audiometer Supplier Hearing Aid Professional Physician's Office http://www.alibaba.com/product-detail/Portable-Digital-Audiometer-Supplier-Hearing-Aid_1233690528/showimage.html

[Accessed 30 May 2015]

[15] Noise-induced Hearing https://www.nidcd.nih.gov/staticresources/health/hearing/NIDCD-Noise-Induced-Hearing-Loss.pdf

[Accessed 30 May 2015]

[16] Git client for Mac and Windows | Atlassian. Available at: https://www.atlassian.com/software/sourcetree/overview?_mid=e0734c30de09bb01fe8ee8625dc35ca4&gclid=CIeZgeqR6sUCFYJZ2wodDFIAxg

[Accessed 30 May 2015]

[17] Guide to Agile Practices. 2015. Guide to Agile Practices. Available at: http://guide.agilealliance.org/guide/storymap.html

[Accessed 31 May 2015]

[18] Pivotal Tracker | Agile Project Management. Available at: http://www.pivotaltracker.com/

[Accessed 31 May 2015]

[19] Arafura Audiology & Alice Audiology | Interpreting an audiogram.

Available at: http://www.arafuraaudiology.com.au/interpretinganaudiogram.html.

[Accessed 31 May 2015]

[20] Available at: http://www.chimehealth.co.uk/web/data/audiogram-hearing-loss-examples-2.pdf. [Accessed 31 May 2015]

[21] Pencil Project. Available at: http://pencil.evolus.vn/. [Accessed 31 May 2015]

[22] Paul Tobin, 2007, PSpice for Digital Signal Processing, Edition 1, Morgan & Claypool Publishers.

[23] AudioTrack | Android Develop. Available at: http://developer.android.com/reference/android/media/AudioTrack.html.

[Accessed 31 May 2015]

[24] Graph View - Summary & Features. [ONLINE] Available at: http://www.android-graphview.org/.

[Accessed 31 May 2015]

[25] Online Tone Generator - Free, Simple and Easy to Use.

Available at: http://www.onlinetonegenerator.com/.

[Accessed 01 June 2015]

[26] JunSoftware109/AndroidProjectHearing · GitHub

Available at: https://www.github.com/JunSoftware109/AndroidProjectHearing.

[Accessed 01 June 2015]